| World Meteorological Organization & Intergovernmental Oceanographic Commission (of UNESCO) GLOBAL OCEAN OBSERVING SYSTEM/OBSERVATIONS COORDINATION GEORP Ship Observations Team Eleventh Session Virtual Session, 13-16 September 2021 | SOT-11/Doc. 13.2 |
| --- | --- |
|  | Submitted by: J.B COHUET 25.08.2021 DRAFT 1 |

AGENDA ITEM 13:	OCEAN BEST PRACTICES

AGENDA ITEM 13.2:	Task Team on Instrument Standards and Satellite Communications, including review of WMO Pub No 8

# SUMMARY

- SUMMARY

During the last intersessional period, the main task of the Task Team on Instrument Standards and Satellite Communications Systems were:

- The collection and update of information concerning instruments, telecommunication and dataflow used by the observing ships. In particular SOT GTS headers and dataflow are now available on the SOT website: https://www.ocean-ops.org/sot/ .

- The monitoring of the telecommunication systems used globally.

- To follow the application of Iridium to be part of GMDSS, and to construe possible consequences for Code41 messages.

- To work in liaison with TT-metadata to create sensors and AWS list from pub47 entries, and to design new telecom tables separating data communication method and telecom services.

- To draft a VOS AWS standards documentation.

- To work on a new version of the JCOMM TR63: Recommended Algorithms for the computation of marine meteorological variables" especially true wind and pressure measurements.

Challenges for the next intersessional phase will be:

- The ocean observing community proposes the development of a new Volume to the Guide to Instruments and Methods of Observation (WMO No. 8). The topic of marine measurements is currently presented in WMO No. 8, Volume III (Observing Systems), Chapter 4: Marine Observation (https://community.wmo.int/activity-areas/imop/wmo-no_8). The TT-ISSC will strongly contribute the new marine volume of the CIMO guide, which will include JCOMM 63 TR documentation.

- To finalize the AWS standard documentation

- To work with Iridium for a Code41 implementation

- To update for national fleets all the sensors and telecommunication metadata in the OceanOps system according to the new metadata fields.

### B. ACTIONS/DECISIONS/RECOMMENDATIONS REQUIRED:

(a)	Adopt draft Action 10.2/1 — To participate in the writting of the new marine volume of WMO n°8 including JCOMM63 TR report in this new volume. (action; TT-ISSC; SOT-12)

Adopt draft Action 10.2/2 — To closely work with Iridium to implement a Code41 on  Iridium GMDSS system (action; TT-ISSC; SOT-12)

Adopt draft Action 10.2/3 —To update observing systems, sensors and telecommunication metadata in OceanOps database. (action; SOT NFP; SOT-12)

Decision 10.2/1 — SOT agree on the new TT-ISSC Terms of Reference (Annex A)

# C. BACKGROUND INFORMATION

Terms of reference

TT-ISSC Terms of reference are updated in Annex A.

Membership

TT-ISSC membership is updated in Annex B.

The TT mailing list is sot-tt-iss@jcommops.org

Instrumentation

A list of Instrument standards guidelines is provided in Annex C.

Annex D lists the sensors and systems used by the different countries for VOS, ASAP and SOOP observations.

## Headers and dataflows

GTS headers for VOS, ASAP and SOOP are now available on the SOT website. https://www.ocean-ops.org/sot/

The VOS dataflows are also available in that document.

The GTS headers are exported in Annex F.

## Telecommunication systems used for VOS observations

Some statistics have been computed overs 6 months in 2020 to analyse the telecom repartition by observations (Figure 1) and by VOS (Figure 2).

Email is now the system of transmission the most used by VOS (60% of active ships), in particular for manual ships and that part is still increasing at it was 45% of the VOS two years ago. In total 33% of the observations are sent by Email.

For AWS, Iridium SBD is the most used system. It represents only 11% of the active ships but producing 42% of the messages sent to the GTS.

The proportion of ships sending messages to the GTS which are not registered in pub47 is decreasing. It was 25% of the active ships in 2018, and they are now only 5% of the ships.

Code41 is still used by many ships (14% to 19% of the VOS considering that the non-registered ships are usually sending their messages in code41). It represents 4% to 8% of the messages distributed on the GTS. Figure 3 and 4 shows 2020 observations done in Code41 for Pub47 registered and non-registered VOS (except those sending observations with the call sign SHIP),and show that this system is still widely used.

Inmarsat Land Earth Station (LES) list is provided in Annex E.

Figure 1 : Telecom used by observation

Figure 2 : Telecom used by VOS

### Figure 3 : Registered VOS observations transmitted in Code41 in 2020

### Figure 4 : Non registered VOS observations in 2020 using Code41 (except SHIP)

## Iridium GMDSS

In early 2020, Iridium was successfully certified by the IMO to offer GMDSS (Global Maritime Distress and Safety System) service to ships. Until then, the service was only provided by Inmarsat.

In October 2020 Iridium contacted WMO to know how to implement the Code41 service, asking for voice numbers in each MetArea. It was answered to Iridium that a voice system is useless for the SOT community and a service to send text messages is needed. That functionality was not implemented on Iridium GMDSS terminals.

Iridium announced that their GMDSS services became operational in December 2020, without SAC41 service implemented.

Some ships will be equipped soon with Iridium GMDSS, a need for a transmission system with this Iridium modem, free of charge for the ship is still needed.

Iridium agreed to discuss about the implementation.

Iridium is working only with one gateway in comparison with Inmarsat LES. They have the possibility to send the messages to a different address according to the ship position.

More discussions with Iridium are needed about a possible Code41 implementation. The expected possibilities and consequences are the following :

| N° | Description | Advantages | Drawbacks |
| --- | --- | --- | --- |
| 1 | According to the position of the ship the message is routed and charged to a Met Service responsible for the area | All the areas are covered All the ships can send a message | All the countries responsible for the area need to have a contract with an Iridium provider and have to be able to send the messages on the GTS. Unfair system: few countries are paying for the whole community |
| 2 | According to the position of the ship the message is routed to a designated destination and charged to a designated Met Service (responsible for the area) or another organization. | All the areas are covered All the ships can send a message | Arrangements/contracts should be made about destinations and who is paying for (all or certain) areas. This can take some time. Depending on arrangements, the system can be more or less fair regarding who is bearing the costs. |
| 3 | National Codes 41 are established. The messages are routed according to the code. | Each country pays for its recruited ship | Risk to lose the non-recruited ships Much paperwork to get a global system working. Very probably only very few national codes will be set. |
| 4 | Global system with only one recipient | Simple system Can handle format evolution like #101 | Need for a reliable data-flow |

## AWS standard

During SOT10, the TT-ISSC was asked to document minimal standards for Ship-borne Automatic Weather Stations (S-AWS), from a specifications perspective, including details on technologies and standards and best practices). the TT-ISSC worked on a draft document available on the TT-ISSC google drive :

https://drive.google.com/drive/folders/188rFUT4stlvjfREtIs_SiqLEuvyYSJ1g

## Algorithm for meteorological variables

JCOMM TR 63 "Recommended Algorithms for the computation of marine meteorological variables" is currently limited to the dew point measurement. Some work has been done to include information on pressure and true wind measurement.

The draft documents available on the TT google drive :

https://drive.google.com/drive/folders/188rFUT4stlvjfREtIs_SiqLEuvyYSJ1g

In the future, this document will be merged with the new marine volume of the WMO N°8 CIMO guide.

## WMO N°8

The ocean observing community proposes the development of a new Volume to the Guide to Instruments and Methods of Observation (WMO No. 8). The topic of marine measurements is currently presented in WMO No. 8, Volume III (Observing Systems), Chapter 4: Marine Observation (https://community.wmo.int/activity-areas/imop/wmo-no_8).

This chapter describes the observing systems employed in Marine Observation, mainly VOS and moored buoys, with brief descriptions of variables observed and best practices primarily focused on marine meteorology. In addition the TT-ISSC is working on the JCOMM TR63:  Recommended Algorithms for the computation of marine meteorological variables. The ocean community is quite diverse with many marine observation focus areas and multiple ocean observing platform networks, each requiring a different set of best practices. The IOC/UNESCO maintains the Ocean Best Practices portal, separate from the WMO No. 8, with measurement practices covering many other ocean-related networks and measurements.

The intention of this proposal is to develop a separate Volume to the WMO No. 8 to encompass the disciplines represented by the ocean observing community for measurement of marine meteorological and oceanographic variables. This approach is better aligned with the content and guidance presented by discipline specific volumes of WMO No. 8 such as:  Volume I. Measurement of Meteorological Variables and Volume II: Measurement of Cryospheric Variables.

Initially, this new volume would extract the Marine Observation material from Volume III, Chapter 4 and place it into a new independent Volume.  The new volume may then be amended to provide more details on the currently described measurement techniques and technologies. The JCOMM TR63 will be incorporated in this volume. Additional guidance and measurement practices may be gradually incorporated into the new volume in the future.

## Actions

The actions from last SOT are listed in the table below:

| Number | Description | Status |
| --- | --- | --- |
| 27 | JCOMM TR 63 "Recommended Algorithms for the computation of marine meteorological variables" is currently limited to the dew point measurement. Other variables have to be described. The JCOMM TR 63 has to be completed with all the other variable measured by the VOS | Draft version available. To be continued and merged with the new volume of the CIMO guide. |
| 28 | Work on metadata extraction from the JCOMMOPS metadata database to create regular synthesis of systems and instruments used in each country. | Pending new metadata scheme implementation |
| 30 | Extend dataflow documentation for VOS to the ASAP and SOOP programmes. | Done for the headers |
| 31 | In order to be precise for each variable’s expected quality of measurement, it is recommended to work on a ship classification per parameter observed. | To be closed. TT agreed not to go that way as the work is too huge. |
| 61 | Document minimal standards for Ship-borne Automatic Weather Stations (S-AWS), from a specifications perspective SOT to document details on the AWS with technologies and standards and best practices. | Draft version available. To be continued. |

ANNEX A

Task Team on Instrument Standards and Satellite Communications Systems

Terms of Reference

- Compile information and provide guidance on existing activities, procedures and practices within SOT relating to instrument testing, standardization and intercalibration, as well as the standardization of observation practices and procedures;

- Using guidance contained in existing guides including the WMO Guides on Instruments and Methods of Observation (WMO-No.8) communicate with manufactures regarding new technologies and recognized equipment problems when requested;

- Prepare dedicated Webpages containing this information, to be made widely available through the SOT web site and linked from other relevant websites;

- Organise inter-comparisons as required by SOT sessions;

- Review and comment relevant WMO Publications when requested to make sure they are kept up to date;

- Evaluate the operational and cost-effective use of satellite data telecommunication systems for the real-time collection of VOS and SOOP data in support of the World Weather Watch, GOOS, and GCOS;

- Evaluate and propose new communication systems for conventional VOS to substitute Code 41 and continue to monitor the cost implications of Code 41;

- Liaise with international organizations (IMO, IMSO) and with telecom providers to take into account expected evolutions relevant to SOT.

ANNEX B

Task Team on Instrument Standards and Satellite Communications Systems

Membership

Jean-Baptiste COHUET (TT Chairperson, France)

Ross BANNISTER (TT ViceChair, New Zealand)

Martin KRAMP (ex officio, SOT TC)

David BERRY (United Kingdom)

Christophe BILLON (France)

Francis BRINGAS (United States)

Joel CABRIE (Australia)

Mardene DE VILLIERS

Darin FIGURSKEY (United States)

Henry KLETA (Germany)

Massaya KONISHI (Japan)

Rudolf KROCKAUER (Germany)

James LUCIANI (United States)

Balakrishnan NAIR (India)

Damian NAPOLES (Netherlands)

Emma STEVENTON (United Kingdom)

Michael POTOCHNEY  (United States)

Peter ROUX (South Africa)

Rene ROZEBOOM (Netherlands)

Adam RYAN (new member, United Kingdom)

Shawn SMITH (United States)

Joaquin TRINANES (United States)

Mayu YAMAMOTO (new member, Japan)

# ANNEX C : Instrument Standards Guidelines

- VOS

  - WMO

    - Guide To Meteorological Instruments and Methods of Observation (WMO-No. 8) Edition 2018

    - TurboWin User guides : https://www.ocean-ops.org/sot/vos/resources.html

  - NMS

    - Australia

        - Port Meteorological Agents Guide

        - TurboWin User Guide

        - TurboWin Setup Manual

    - Germany

      - Port Met Officers Work Instruction

      - Marine Observers Guide

      - TurboWin manual (+ DWD own help doc to TurboWin)

    - Hong Kong, China

        - Guidance Notes on Port Meteorological Services

        - Marine Observers Handbook

        - UK Met.O. 740

    - Japan

        - Guide to Weather Observations for Ships (JMA)

        - Guide to Ships Weather Reports (JMA)

        - Manual on Port Meteorological Services (JMA, in Japanese)

    - United Kingdom

        - Marine Observers Handbook

        - TurboWin user/installation guideline

        - UK Met.O.740 (Only provided on request from ship)

    - United States of America

        - Military Specification MIL-B-17089

        - National Weather Service NWS G101 – SP004

        - National Weather Service NWS G222 – SP002

        - NWS Instruction 10-201 (Feb 24, 2012) http://www.nws.noaa.gov/directives/sym/pd01002001curr.pdf

- SOOP

  - IOC

      - Guide to IGOSS (now JCOMM) Data Archives and Exchange (BATHY and TESAC) - IOC Manual and Guides No.1

      - Guide to Operational Procedures for the Collection and Exchange of IGOSS (now JCOMM) Data - IOC Manual and Guides No.3

      - IGOSS (now JCOMM) Plan and Implementation Programme - IOC Technical Series No. 43

      - Best Guide and Principles Manual For The Ships Of Opportunity Program (SOOP) and Expendable Bathythermograph (XBT) Operations

      - GO-SHIP Repeat Hydrography Manual

http://www.go-ship.org/HydroMan.html

  - NMS

      - Australia

        - Devil XBT User Manual

- ASAP

  - WMO

    - No guidance available at this time.

  - E-ASAP

    - No guidance available at this time.

  - Germany

    - ASAP-Manual for sounding procedures

  - Japan

    - Guide to Upper-Air Observation (JMA; in Japanese)

# ANNEX D

Instrument Standards Equipment Status Report

## Conventional VOS

## Barometer

| BAROMETERS ON CONVENTIONAL VOS |  |  |  |  |
| --- | --- | --- | --- | --- |
| National VOS | Barometer | Barometer Type | Barometer Setting | Type of Correction Tables Used |
| Argentina | CS100 Setra. Model 278 | Digital Digital | Station Level Station Level | Height |
| Australia | Vaisala PTB220 Vaisala PTB330 | Digital Digital | Station Level Station Level | Height Height |
| Croatia | Barigo Fisher SUNDO | Ship's Aneroid Ship's Aneroid Ship's Aneroid | MSL MSL MSL | NIL NIL NIL |
| Ecuador |  | Aneroid | MSL | NIL |
| France | Vaisala PTB220 | Digital | Station Level | Height |
| Germany | Fuess Vaisala PTB330 | 15PM Digital | MSL MSL | NIL NIL |
| Greece | Belfort | Aneroid | Station Level | Height / Temperature |
| Hong Kong, China | Hisamatsu Mintaka Duo | Precision Aneroid Ship's Aneroid Digital | Station Level Station Level Station Level | U.K. Met. O. 740 U.K. Met. O. 740 Temp/Height (TurboWin) |
| Iceland | Fuess Vaisala PA11 | Ship's Aneroid Digital | MSL MSL | Air Pressure Dependent |
| Ireland |  | Ship's Aneroid Aneroid | MSL MSL | NIL NIL |
| Japan |  | Aneroid Digital | Station Level Station Level | Pressure/Temperature Height Pressure/Temperature Height |
| Netherlands | Fuess Vaisala PTB220 Vaisala PTB330 | Aneroid Digital Digital | Station Level Station Level Station Level | Temp / Height (TurboWin) Temp / Height (TurboWin) Temp / Height (TurboWin) |
| New Zealand | Fuess Vaisala PTB330 Vaisala PTB220 | Aneroid Precision Aneroid Digital Digital | MSL Station Level Station Level Station Level | NIL Instrument & Height Height Height |
| Norway | Vaisala PTB220 | Digital | MSL | NIL |
| Singapore | PAB MK2 M2236 |  | MSL | U.K. Met. O. 740 |
| South Africa | Fuess Viking | Aneroid Aneroid | MSL MSL | NIL NIL |
| Sweden | Vaisala PTB220 | Digital | Station Level | Height or TurboWin |
| United Kingdom | Negretti & Zambra PAB MK2 Vaisala PTB330 | Precision Aneroid Barometer Digital | Station Level Station Level | NIL Temp / Height (TurboWin) |
| United States | Belfort Fischer Meteograf Aquatech DBX1 Mintaka Duo | Aneroid Aneroid Digital Digital Digital | MSL MSL MSL MSL Station Level | NIL NIL NIL NIL NIL |
| NOTE: For Ships using TurboWin, the Height correction is applied by the software. |  |  |  |  |

## Barograph

| BAROGRAPHS ON CONVENTIONAL VOS |  |  |  |
| --- | --- | --- | --- |
| National VOS | Barograph | Barograph Type | Barograph Setting |
| Australia | Negretti & Zambra Vaisala PTB220 Vaisala PTB330 | Open Scale (7 Day) Digital Digital | Station Level Station Level Station Level |
| Croatia | KOMPAS | Open Scale | MSL |
| Ecuador |  | Micro-Barograph | MSL |
| France | None |  |  |
| Germany | Mueller 78A Lambrecht 290 Vaisala PTB330 | Small Scale Small Scale Digital | MSL MSL MSL |
| Greece | Belfort | Open Scale (4 Day) | Station Level |
| Hong Kong, China | Fischer Sato Hisamatsu Isuzu Seisakusho OTA Mintaka Duo | Small Scale Small Scale Small Scale Small Scale Small Scale Digital | Station Level Station Level Station Level Station Level Station Level Station Level |
| Iceland | None |  |  |
| Ireland |  | Open Scale (7 Day) | MSL |
| Japan |  | Open Scale (1 Day) Open Scale (7 Day) | Station Level Station Level |
| Netherlands | Fuess Vaisala PTB220 Vaisala PTB330 | Aneroid Digital Digital | Station Level Station Level Station Level |
| New Zealand | Vaisala PTB330 | Open Scale Digital Display | MSL Station Level |
| Norway | Vaisala PTB220 | Digital Display | MSL |
| Singapore |  | Open Scale MK3 | MSL |
| South Africa | Mason |  | MSL |
| United Kingdom | CASELLA MK2Fischer MK3 Vaisala PTB330 | Open Scale (7 Day) Small Scale Digital | Station Level Station Level Station Level |
| United States | Aquatech DBX1 Meteograf Mintaka Duo | Digital Digital Digital | MSL MSL Station Level |

## Transfer Barometer

| VOS TRANSFER STANDARD BAROMETERS ON CONVENTIONAL VOS |  |  |
| --- | --- | --- |
| National VOS | Barometer | Frequency of Barometer comparison |
| Australia | Vaisala PTB220B | 4 monthly |
| Equador | OACI mercury | 6 monthly |
| France | None |  |
| Germany | Vaisala PTB330 Druck DPI 740 | 6 monthly 6 monthly |
| Greece | Belfort Aneroid | 3 - 6 monthly |
| Hong Kong, China | Vaisala PTB330 | 9 monthly |
| Japan | Vaisala Digital Barometer Paroscientific | 6 monthly |
| Netherlands | Fuess Aneroid Vaisala PTB220 Vaisala PTB330 | 12 monthly 12 monthly 12 monthly |
| New Zealand | Vaisala PTB220 | 12 monthly |
| Norway | Digiquartz | 12 monthly |
| South Africa | Vaisala PA11 & Precision Aneroid | 3 - 6 monthly |
| Sweden | Vaisala PA11A | 12 monthly |
| United Kingdom | Vaisala PTB330TS (includes temp/humidity with handheld device) | 12 monthly |
| United States | Digiquartz | 6 monthly |

## Air thermometers

| THERMOMETERS ON CONVENTIONAL VOS |  |  |  |
| --- | --- | --- | --- |
| National VOS | Thermometer | Thermometer Type | Thermometer Fluid |
| Australia | AMA Zeal PT100 | Liquid-in-glass Liquid-in-glass Digital | Hg Hg |
| Germany | Sling Eigenbrodt Rotronic Hygropalm | Liquid-in-glass Digital | Hg |
| Greece | Schneider | Liquid-in-glass | Hg |
| Hong Kong, China |  | Liquid-in-glass | Alcohol |
| France | Vaisala HMP34 | Digital |  |
| Netherlands | Schneider | Liquid-in-glass | Alcohol |
| Norway | PT100 |  |  |
| Sweden | PT100 Testo 950 |  |  |
| United Kingdom | Zeal 2C Rotronic Hygropalm | Liquid-in-glass Digital | Alcohol |
| United States | Zeal P2505 Extech RH300 PHT-771 | Mason Hygrometer Digital Digital | Glycol |

## Sea Surface temperature sensors

| SEA SURFACE TEMPERATURE ON CONVENTIONAL VOS |  |  |  |
| --- | --- | --- | --- |
| National VOS | Sensor | Sensor Type | Sensor Scale C/F |
| Australia | Sea thermometer | Ship’s intake Bucket (UK) Hull contact sensor | C C C |
| Germany | Sea thermometer | Bucket Ship's intake Hull contact sensor | C C C |
| Greece | Sea thermometer | Ship's intake | C |
| Hong Kong, China | Sea thermometer | Ship's intake | C |
| Netherlands | Sea thermometer | Ship's intake Bucket (alcohol or mercury) | C |
| Norway | Sea thermometer | Hull contact | C |
| United Kingdom | Sea thermometer | Bucket Ship’s intake Hull contact sensor | C C C |
| United States | Sea thermometer | Bucket Model 2170-TX | Either (ship Dependent) |

## Automatic VOS

| Automatic Weather Stations on VOS |  |  |  |
| --- | --- | --- | --- |
| National VOS | Type of AWS | Communication Method | Manual Entry Facility |
| Argentina | Campbell Scientific | Inmarsat C |  |
| Australia | Vaisala Milos 500 AWS Techsas | Inmarsat C (Data Mode) Inmarsat C (Data Mode | Yes No |
| Brasil | VAISALA MAWS410 VAISALA MAWS430 | HF Radio/Satellite HF Radio/Satellite | Yes Yes |
| Canada | AVOS – AXYS Technologies | Inmarsat C Iridium | Yes |
| China | XZC6-1 XZC2-2SA DJQ-1 | Inmarsat C CDMA | Yes |
| France | BATOS MERCURY EUCAWS | Inmarsat C (Data Mode) & Iridium SBD Iridium SBD Iridium SBD | Yes Yes Yes |
| Germany | Vaisala Milos 500 AWS Ship's own datalogger SCAWS EUCAWS | eMail / Meteosat Inmarsat / Iridium eMail Iridium SBD | Some Yes No Yes |
| Hong Kong, China | AMOS MetOcean drifting buoy onboard VOS | Iridium Iridium | No No |
| Indonesia | BATOS | Iridium SBD | Yes |
| Ireland | Vaisala Milos AWS | Meteosat | No |
| Japan | Koshin Denki Kogyo Co., Ltd (Japan) Surface Meteorological observation (SMet) system | Email Email | Yes Yes |
| Netherland | EUCAWS | Iridium SBD | Yes |
| New Zealand | iSTAR mSTAR-SHIP | Inmarsat GPRS Cell | Yes No |
| Norway | PC with QLC50 EUCAWS | VSAT with Iridium | Yes |
| Portugal | EUCAWS | Iridium SBD | Yes |
| Russia | GM6 | Inmarsat C | Yes |
| South Africa | Vaisala Milos 520 | Inmarsat C | Yes |
| Spain | Vaisala MAWS 410 EUCAWS Campbell Scientific CR1000X | Inmarsat C by email Iridium VSat X-band | No Yes No |
| South Africa | Homdade from Campbell | eMail | Yes/No |
| United Kingdom | AMOS AMOS2X | Iridium Iridium | No No |
| United States | SEAS-AutoImet Ship’s own datalogger | SEAS eMail | Some No |
| SENSOR FOR VOS AUTOMATED SYSTEMS |  |  |  |
| National VOS | Parameter | Sensor | Calibration interval |
| Argentina | Pressure Combined Humidity and Temp Wind | CS100 Setra. Model 278 Campbell HC2S3 Campbell 05108-L |  |
| Australia | Pressure Air temperature Humidity Wind SST | Vaisala PTB220 Rosemount ST2401 Vaisala HMP45D Vaisala HMP155 Vaisala WAV151 & WAA151 Seabird SBE48 | 6 months 6 months 6 months 24 months 24 months |
| China | Pressure Combined Humidity and Temp Wind SST | R.M.Young 61302 Setra278 R.M.Young 41382 Vaisala HMP155A Hydrowise XFY3-1 R.M.Young 05103 Vaisala WMT702 Everst Interscience 4000.3ZL TRDI TS-N |  |
| France | Pressure Air temperature Humidity Wind SST | Vaisala PTB210 Vaisala PTB220 Vaisala PTU200 Vaisala PTB330 PT100 HMP110 HMP45D Vaisala HMP35DE Vaisala HMP45D Vaisala HMP110 Gill Windsonic WS2 Vaisala WMT52 PT100 intake Hull contact PT100 | 2 years 12 months 12 months 12 months |
| Germany | Pressure Air temperature Humidity Combined Humidity and Temp Wind SST | Vaisala PTB (220 + 330) PT100 Vaisala HMP45D E+E33 Lufft Ventus (ultrasonic) Thies Cup and Vane PT100 | 18 months 5 years 12 months 12 months 24 months 24 months 10 years |
| Hong Kong, China | Pressure Air temperature | PTB110 Build in temperature sensor of MetOcean drifting buoy | 12 months |
| Indonesia | Pressure Air temperature Humidity Wind SST | Vaisala PTB330 Vaisala HMP155 Vaisala HMP155 Gill Windsonic WS2 Hull contact PT100 |  |
| Japan | Pressure Air temperature Humidity Wind SST | Vaisala PTB 220 Setra Systems MODEL 370 CHINO CORPORATION R005-341 Vaisala HMP155 Vaisala HMT3303 JM Vaisala HMP155 Koshin KVS-400-J Koshin KS-5900 CHINO CORPORATION RFN1-0 Koshin RFN2-0 | 10 years 12 months 12 months 5 years 12 months |
| Netherlands | Pressure Combined Humidity and Temp Wind | Vaisala PTB 220 / 330 E+E33 OMC-116 | 38 months 8 months |
| New Zealand | Pressure Air temperature Humidity Wind | Vaisala PTB330 Vaisala HMP155 Vaisala HMP155 Vaisala WAA151 &WAV151 Vaisala WMT702 |  |
| Spain | Pressure Air Temperature Humidity Wind SST | Vaisala PTB220 Vaisala PTB330 Setra CS100 Vaisala HMP155 Vaisala HMP155 Vaisala WMT700 Vaisala WMT702 MetSENS200 Hull contact Vaisala Hull contact DWD |  |
| South Africa | Pressure Air temperature Humidity Wind | Vaisala PTB110 Vaisala HMP155 Vaisala HMP155 Gill Observer 2 |  |
| United Kingdom | Pressure Combined Humidity and Temp | Vaisala PTB110 Vaisala HMP110 | ~3 years ~1 year from installation |
| United States | Pressure Air temperature Wind SST | Young 61201 Vaisala PTB330 Young 26800 Young 41372VC Young Vaisala WMT700 IMET Wind sensor Integrated Seabird |  |

## SOOP

## Expendable BathyThermograph (XBT)

| XBT Probe |  |
| --- | --- |
| National SOOP | Equipment Type |
| Australia | Sippican |
| France | Sippican (Deep Blue, T7 and Fast Deep) |
| Italy | Sippican (T4, T5, T7, T10, DeepBlue) |
| Japan | TSK(T-7) |
| United States | Sippican (DeepBlue and FastDeep) |

| XBT Recorder |  |
| --- | --- |
| National SOOP | Equipment Type |
| Australia- BOM Australia- CSIRO | Quoll XBT Quoll XBT |
| France | Devil XBT Sippican MK21 USB (DAQ) |
| Italy | Sippican MK21 USB (DAQ) Sippican MK21 USB (Ethernet) |
| Japan | TSK (MK150) |
| United States | Sippican MK21 ISA Sippican MK21 USB (DAQ) Sippican MK21 USB (Ethernet) |

| XBT Launcher |  |
| --- | --- |
| National SOOP | Equipment Type |
| Australia | Sippican Hand Launcher (modified LM3A) |
| France | Sippican Hand Launcher (LM3A) |
| Italy | Sippican Hand Launcher (LM3A) |
| Japan | TSK(Hand-held Launcher) |
| United States | Sippican Hand Launcher (LM3A) AOML Autolauncher (AOML) Scripps Autolauncher (SIO) |

| XBT Transmission |  |
| --- | --- |
| National SOOP | Equipment Type |
| Australia-BOM Australia-CSIRO | Iridium |
| France | Argos |
| Japan | email |
| United States | T&T Sailor 403026S Mini-C transceiver Iridium NAL SAF4070-IG Iridium NAL A3LA-XG Iridium NAL A3LA-RG Garmin GPS 18X PC Furuno GPS Navigator GP-32 |

## Thermosalinograph (TSG)

| Thermosalinograph (TSG) |  |
| --- | --- |
| National SOOP | Equipment Type |
| France | Seabird 21 TSG |
| Japan | Seabird 38 Remote Temperature Sensor Seabird 45 Micro TSG |
| United States | Seabird 21 TSG Seabird 38 Remote Temperature Sensor Seabird 45 MicroTSG |

| TSG Transmission |  |
| --- | --- |
| National SOOP | Equipment Type |
| France | Inmarsat C |
| United States | Iridium NAL A3LA-RG Garmin GPS 18X PC |

## Conductivity, Temperature, and Depth (CTD)

| Conductivity, Temperature, and Depth (CTD) |  |
| --- | --- |
| National SOOP | Equipment Type |
| Japan | Seabird 911+ |
| United States | Seabird 19 Seabird 25 Seabird 911+ |

## Expandable Conductivity, Temperature, and Depth (XCTD)

| Expandable Conductivity, Temperature, and Depth (XCTD) |  |
| --- | --- |
| National SOOP | Equipment Type |
| Italy | TSK XCTD-1 |
| United States | Sippican XCTD-1 Sippican XCTD-3 |

## Acoustic Doppler Current Profile (ADCP)

| Acoustic Doppler Current Profile (ADCP) |  |
| --- | --- |
| National SOOP | Equipment Type |
| Japan | RD Instruments |
| United Stated | RD Instruments |

## Partial Pressure of CO2 (pCO2)

| Partial Pressure of CO2 (pCO2) |  |
| --- | --- |
| National SOOP | Equipment Type |
| Australia | CSIRO |
| France | Craig Neil General Oceanics 8050 |
| Japan | Nihon ANS system (LI-COR LI700) Los Gatos Research (Off-Axis ICOS) |
| United States | General Oceanics 8050 |

## Moving Vessel Profiler

| Moving Vessel Profiler |  |
| --- | --- |
| National SOOP | Equipment Type |
| United States | Brooke |
| United States | Scripps |

- ASAP

| ASAP TYPES and COMMUNICATIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| National ASAP |  | Container | Sounding Equipment | SATELLITE TRANSCEIVER |
| E-ASAP | Denmark | 10ft container Built-In launcher 10ft container | Vaisala MW41 Vaisala MW41 Vaisala MW41 | A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium |
|  | E-ASAP | Deck launcher Deck launcher Deck launcher Deck launcher Deck launcher 10ft container | Graw GS-E Graw GS-E Graw GS-E Graw GS-E Graw GS-E Vaisala MW41 | A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium |
|  | France | Deck launcher Deck launcher Deck launcher Deck launcher | MODEM SR10 MODEM SR10 MODEM SR10 MODEM SR10 | A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium |
|  | Germany | 10ft container 20ft container Deck launcher Deck launcher | Vaisala MW41 Vaisala MW41 Vaisala MW41 Vaisala MW41 | A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium A3LA-RM Iridium |
|  | Spain | 10ft Container | Vaisala MW21 | A3LA-RM Iridium |
| Germany (AWI) |  | Built-in (manual) | Vaisala MW31 | email (ship system) |
| Japan |  | 10ft Container 20ft Container | Vaisala MW41 Vaisala MW41 | Email Email |

ANNEX E : Land Earth Station list

ANNEX F

GTS headers list

The GTS headers are now available on SOT Website : https://www.ocean-ops.org/sot/ in the headers/dataflows document.

GTS headers are listed below :

VOS

| Source of observation | GTS header for FM 13 SHIP | BUFR template | GTS header for BUFR |
| --- | --- | --- | --- |
| Code41 sent to LES Aussaguel (FR) | No | TM 308009 | ISS[A-L]02 LFPW |
| FM13 data flow sent from KNMI to MF (among them Code41 sent to LES Burum(NL), | No | TM 308009 | ISS[A-L]04 LFPW |
| Code41 sent to LES Gunillhy (UK) UK VOS (Turbowin via email) | SXVX88 EGRR | TM 308009 | ISS[A-L]ii EGRR |
| Code41 sent to LES Southbury (US) | S[MIN]V[A-X] ii 01-19 KWBC | TM 308009 | ISS[A-L]01-19 KWBC |
| Code 41 sent to Thermopylae (GR) | no | TM 308009 | ISSF01 LGAT ISSF20 LGAT |
| Australian VOS (AWS, Turbowin) | SMV[A-J]01 AMMC S[I,N]V[A-J]21 AMMC | No | No |
| Canadian VOS (AWS, Turbowin) | SNVD01 KWBC | No | No |
| French AWS (Batos, Baros) + Half compressed (APR, email, Obs2server) from Esurfmar VOS | No | TM 308009 | ISS[A-L]01 LFPW |
| French Mercury AWS | No | TM 308009 | ISS[A-L]03 LFPW |
| Eucaws stations collected by MF | No | TM 308009 | ISS[A-L]05 LFPW |
| German VOS | No | TM 308009 | ISS[A-L]01 EDZW |
| Hong Kong VOS | SIV[B,E,X]20 VHHH SMV[B,E,X]01 VHHH | TM 308009 | ISS[A-D,I-L,X]01 VHHH |
| Indian AWS | SMVX02 DEMS | No | No |
| Japanese VOS (Obsjma, Turbowin, AWS) | S[MIN]V[BDEX]ii RJTD | TM 308009 | ISS[A-D,I-L]01-02 RJTD |
| Spanish AWS (Esperanza del mar) | No | TM 308009 | ISSA01 LEMM (SHIPs 00, 06, 12 and 18UTC), ISSA22 LEMM (SHIPs 03, 09, 15 and 21UTC), ISSA21 LEMM (hourly SHIPs) |
| Spanish AWS (Hesperides) | No | TM 308009 | ISSX01 LEEM |
| Sweden | No | TM 308009 | ISS[A-L]01ESWI |
| UK Vos (AMOS, turbowin to server) | S[MIN]V[A..X] ii EGRR | TM 308009 | ISS[A-L]ii EGRR |
| Irish Turbowin VOS | No | TM 308009 | ISSA[20]1 EIDB |
| US VOS (Turbowin, Anverseas) | S[MIN]V[A-X] ii 01-19 KWBC | TM 308009 | ISS[A-L]01-19 KWBC |
| US VOS (Turbowin, Anverseas) | S[MIN]V[A-X] ii 01-19 PANC | TM 308009 | ISS[A-L]01-19 PANC |

ASAP

| Source of observation | BUFR template | GTS header for BUFR |
| --- | --- | --- |
| E-ASAP | 309052 | IUKX40/IUSX40 EDZW |
| DE-ASAP | 309052 | IUKX40/IUSX40 EDZW |
| FR-ASAP | 309052 | IUKX40/IUSX40 LFPW |
| DK-ASAP | 309052 | IUKX40/IUSX40 EKMI |
| ES-ASAP | 309052 | IUKX40/IUSX40 EDZW |

SOOP

| Source of observation | System | TAC message | TAC Header | BUFR template | GTS header for BUFR |
| --- | --- | --- | --- | --- | --- |
| Australia (BOM, CSIRO, RAN) | XBT | BATHY | SOV[A-F,J]01 AMMC | TM 315004 | IOS[N/S]01 AMMC |
| Brazil | XBT | BATHY | SOV[A, C, D, J]01 SBBR |  |  |
| Canada | CTD | TESAC | SOVD02 CWOW |  |  |
| Canada | XBT | BATHY | SOVD02 CWOW |  |  |
| France (IFREMER) | XBT | TESAC | SOVX05 LFPW | TM 315004 | IOSX01 LFVX |
| France (IFREMER) | CTD | TESAC | SOVX06 LFPW | TM 315007 | IOSX01 LFVX |
| France (IFREMER) | TSG | TRACKOB | SOVX06 LFPW |  |  |
| France (IFREMER) | Miniloggers |  |  | TM 315002 | IOSX01 LFVX |
| Japan | CTD | TESAC | SOVX02 RJTD | TM 315007 | IOS[A-D,I-L]nn RJTD |
| Japan | XBT | BATHY | SOVX01 RJTD | TM 315004 | IOS[A-D,I-L]nn RJTD |
| Japan | TSG | TRACKOB | SOVX12 RJTD | TM 308010 | IORX01-03 |
| USA (NOAA/AOML, SIO) | XBT | BATHY | SOVX01 KWBC | TM 315004 | IOSX01 KWBC |

An Action/Decision  is an item directly related to SOT and on which SOT can action or decide directly. Details on rational for the action/decision should be included in the Background section.